DIALER 1 and DIALER 2

DOCUMENTATION

The DIALER programs  are MODEM communications programs  for use with

the TI-PC and the TI-PPC. The programs were  originally published in "TI

Professional   Computing"   magazine  but  the   author  has  authorized

distribution and use of  the programs. DIALER 1 is for  use with a TI-PC

using  an internal  MODEM; DIALER  2 is  for use  with a  TI-PC using an

external MODEM such as a Hayes Smartmodem.

The DIALER  1 program is  currently written for  a computer with the

MODEM set up  to operate in Communication  Port Number ONE. This  Comm 1

setup is controlled by  "jumpers" on control pins on the MODEM board. If

your MODEM  is set to  operate on any  other port you  will have to make

some changes in the DIALER 1 software to make it compatible.

The DIALER  2 program is  currently written for  a computer with the

Serial  Port (which will be  connected to the external  MODEM) set up to

use  Communications  Port  Number  ONE.  This  Comm  1 selection is also

controlled  by "jumpers"  located on  the Serial  board. If  your Serial

board is not set up to use the Serial Port on Comm 1, then you will have

to make some changes in the DIALER 2 software to make it  compatible. If

you are not using a Hayes Smartmodem, you may also have to make a change

in the MODEM "set-up" command.

These programs had many errors in the original versions published. I

think the errors have been corrected and I think both should now operate

satisfactorily. I have tested the DIALER 1  and found it to be effective

but a very basic  and simple program. I have not tested the DIALER 2 but

I believe it should perform similarly.

These  are "no  frills" programs  but they  are written in BASIC and

could be easily adapted to special purposes you may have. They are  only

capable  of communications  using ASCII  characters. You can communicate

from  the  keyboard  and  you  can  transmit  and receive files that are

"ASCII"  files.  ASCII  files  are  "text"  files,  or word processor or

spreadsheet  files that have  been "exported" in  ASCII "text" format or

BASIC  programs  that  have  been  saved  with  the <SAVE"(filename)",A>

format.  These  programs  will  not  transmit or receive "binary" files.

(This documentation file is an ASCII file.)

Both programs can be selected  directly from the UTILITY Menu.  When

selected they will ask  if you want to "auto-dial" the  number. You will

then be offered  a Menu of stored  Bulletin Board numbers that  you call

frequently. If you  answered "N" to  the auto-dial question,  select "0"

(zero) and press <RETURN> and you will be asked for the phone number and

for the speed (300 or  1200 baud) that you want  to use. If you wish  to

auto-dial, after the "Y" you can select a number from the Menu. The Menu

will be reprinted placing a "-" in place of the "*" before the  selected

number. Then depress the "0" (zero) to initiate the dialing.

The  DIALER will dial  the number and  will print "carrier" when you

get a computer carrier.  If you are auto-dialing it will also give you a

warbling audio signal,  too. At that  time hit the  <RETURN> key two  or

three  times  slowly  to  permit  the  other computer to match your send

signal. If the number (Bulletin Board, etc) you are calling has a log-on

procedure you can "pre-set" up to three items in the file to be recalled

by depressing the <F1>, <F2> or <F3> key. These could include your name,

account  number,  or  a  pass  word.  During  the  communication you can

transmit  a file  by selecting  the <F5>  key, entering  the filename to

transmit,  then pressing the  <F5> key again.  To download a file select

the  <F6> key,  enter the  filename into  which you  wish the file to be

saved, then depress the <F6> again. To end a transmission or  reception,

depress the  <F5> or <F6> respectively, again.  Remember, the files must

be in ASCII format.

Ending the  session with  these programs  is awkward.  If the  other

station   drops  off  the  line,  the  DIALER  software  will  open  the

communication port again  and redial the  same number. To  stop this you

can "break" the  program by depressing the <SHIFT>  key and pressing the

<BRK/PAUS> key.  To make sure the phone line  isn't still open then type

<CLOSE 1><RETURN>. Use the same  procedure if you wish to  terminate the

call - break the program and close the port.

Numbers that you  can Auto-dial must be placed  into data statements

in lines 1110  to 1260. More could  be added, the limit  will be room on

the display. After  adding or deleting numbers, change the number in the

NBOARDS  statement in line 1080 to  correspond to the number of entries.

Each entry should contain six items. First is the name of the entry, the

phone number, the speed (300  or 1200), and the three log-on  entries to

be called by F1, F2, and F3.  Note that all items except the speed  must

be  in quotation marks. Note the sample entry at line 1160. When calling

Wheaton Plaza  you are asked  the number of  lines on your  display, the

number  of  columns,   and  whether  you  have   answered  their  survey

questionnaire. I have set this entry up to provide those answers quickly

by depressing the three function keys, one at a time, when asked.

SET-UP CHANGES FOR DIALER 1

If you  are NOT using Comm  Port 1 for your  Internal MODEM you must

change  several  statements  in  the  program.  In  line 1770 you should

substitute "Comm2, Comm3, or Comm4", whichever  one you have selected on

your  MODEM board, for the  "Comm1" in line 1770.  You will also need to

change the hexidecimal addresses for the control channels of the MODEM's

microprocessor. The two addresses  for Port 1 are "&HE4"  and "&HE6". If

you  are  using Port    2, change  these addresses   to read  "&HEC" and

"&HEE" respectively.  If you are  using Port 3  the addresses are "&HF4"

and  "&HF6".  For  Port  4  use  addresses  "&HFC" and "&HFE". There are

several lines between 1780 and 2320 where these changes must be made.

DIALER 1 SET-UP FOR 300 BAUD MODEM

The above instructions are valid for all internal MODEMS. If you are

using  a 300 Baud  (only) internal MODEM  you must make three additional

changes in the software as follows:

1805 OUT  &HE7,0       (This  is for  COMM1, if  you are  using

COMM2, 3, or 4  it would be "&HEF, &HF7,

or &HFF")

1830 PRINT #1, "A";NUMBERS$(N);"X";:GOSUB 2770:REM send out the

phone#

1840 REM

SET-UP CHANGES FOR DIALER 2

If you are NOT  using Comm Port 1  for your External MODEM  you must

also  make the changes  as noted above.  However, for the External MODEM

there are  fewer changes. There  is one change  in line 1700  and two in

line 2110. Addresses are the same as DIALER 1 above but you only address

the higher  channel, "&HE6", "&HEE", "&HF6", and  "&HFE". If you are not

using a Hayes  Smartmodem, you should check the  MODEM set-up command in

line 1720 to ensure that it is correct for your MODEM.