********************************************************************

CLEAN II

********************************************************************

A simple program to make full use of cleaning disks

such as those sold by 3M and Verbatim.

Format of command:

X>CLEAN [drive:]

If no drive is specified, CLEAN assumes you want to work on the

default  drive, else drive  a:, b:, etc. When  it is through you are

asked if you want to  clean any other drives and, if so, prompts for

the drive  letter. This  works on  single or  double sided  and quad

density   drives  without   modification  and   naturally  will  not

accomplish too much on your fixed disk. No harm  will come, however,

if  it is  run on  your fixed  or floppy  disk as it uses the PC-DOS

absolute sector READ to move the head around the disk.

The reason  you use a cleaning utility rather than using DIR or

any  other command to get the disk  moving is that since the disk in

the drive is paper, the head will never get past Track 0, where  DOS

goes for the directory, in  order to try to accomplish your command.

In other words you  will never use more than 2.5%  of the surface of

your  cleaning disk.  CLEAN knows  there is  a cleaning  disk in the

drive, ignores the DOS error  message that the READ has  failed, and

tries to read another sector on another  track. This way the head is

moved about the disk and the disk is more effectively cleaned.

Cleaning your disk drive is good preventive medicine, but don't

overdo  it.   A  drive  that  is  in  heavy use probably needs to be

cleaned once a week; but, on  the average once every other month  is

plenty.

This  is the  second version  of this  program.  At  the time I

wrote CLEAN I only  had single-sided drives and was not able to test

CLEAN  with a double-sided drive.  CLEAN  now operates closer to the

recommended 30  seconds for cleaning a disk.   It also uses a random

factor to start  its journey across the surface on different tracks.

Since it uses only the most basic of MS-DOS functions, it should run

on any MS-DOS computer.

Program by:

Marty Smith SOURCE ST2259 / COMPUSERVE 12155,1214

310 Cinnamon Oak Lane     (713) 661-1241 [Office]

Houston, TX 77079?        (713) 464-6737 [Home]

[At my request, Marty has modified the program so  that it will

work on any drive letter, so those of us with add-on quad disks

can be next to godliness,  too.  Don't try sticking a  cleaning

disk  in  your  RAM  drive,  though!   R.  Jueneman/SYSOP/(703)

237-4322]