I've noticed lately that many terminals and computers have the

ability to blank their screens after a period of inactivity

in order to keep the phosphor from being eaten away.  I thought

this was a silly frill until I blanked my monochrome display

in reverse video and looked closely.  And I've had it less

than a year!

The Monblank.com program disables the video on a monochrome display

after five minutes of inactivity.  Everything disappears except the

flashing cursor.  The display is re-enabled when either a video I/O

or keyboard interrupt occur.  Noth that some programs (for example

Wordstar 3.0, or Monoclok.com), access the video ram directly and

thus circumvent the video I/O interrupt.  If you are using such a

program and don't touch the keyboard for five minutes, the screen

will blank even if there is video I/O going on.  If this happens,

just press any key, even a shift key, and the display will return.

Run the program once in DOS and it will stay resident until power

down or re-boot.

To adjust the time delay, get into debug with the program and patch

the bytes at 0129H and 014AH with a new value.  The delay will be

about 14 seconds times the number at these locations.

For Colblank, use locations 0131H and 0160H.  -Andy Nicholson

Rich Winkel