The Stack and Other Registers

Section II.

Complex calculations can be done easily with TexSolver-10 because

intermediate results are automatically stored and returned in the

"stack" registers.  This significantly reduces the number of keystrokes

needed to evaluate mathematical expressions.  This section explains the

usage of the stack registers and also the LAST X, storage, and LAST OP

registers.#

The Stack

The stack consists of registers X, Y,Z, and T. The stack contents are

always displayed.  Any number keyed in or resulting from execution of a

function is placed in the X register. Depending upon the type of

operation performed, the stack contents will lift, drop, or remain the

same.  For example, number entry lifts the stack, one-number functions

do not alter stack positions, and two-number functions cause the stack

to drop.#

Stack Functions

ENTER. After a number is keyed in, pressing ENTER indicates that one

number is finished being keyed in and prepares the calculator to accept

the next one.  When ENTER is pressed, the stack is lifted, that is, X

is copied to Y, Y to Z, Z to T, and T is lost.#

R# (Rotate Down, SHIFT-P) rotates the stack register contents down one

register. T is copied to Z, Z to Y, Y to X, and X to T.#

X##Y (X exchange Y, SHIFT-O) exchanges the numbers in the X and Y

registers.#

One-Number Functions and the Stack

One-number functions operate only on the number in the X register.  The

Y, Z, and T registers are not altered when a one-number function is

executed.  Examples of one-number functions are 1/X and X^2.#

Two-Number Functions and the Stack

Functions that operate on two numbers require that the first number be

positioned in the Y register and the second number in the X register.

The result is placed in the X register and the stack drops after the

function is executed, that is, T is copied to Z, and Z to Y.  This

facilitates chain calculations.  Examples of two-number functions are +

and Y^X.#

LAST X

In addition to the four stack registers, there is also a separate

register, the LAST X register.  When most functions are executed, a

copy of the previous X register value is stored in the LAST X

register.  If desired, this value can then be restored to the X

register using the LAST X function (CTRL-X).  This feature allows the

number to be reused without reentering it.  Examples of operations that

store the old X value in LAST X are * and �X. Examples of operations

that do not are ENTER and STO, since X is not modified.#

Storage Register Operations

In addition to the stack and LAST X registers, 100 separate data

storage registers, numbers 00 to 99, are available.#

STO (Store).  When this function is executed (SHIFT-I), you are

prompted to key in a two-digit number (00 to 99) to indicate a storage

register.  The X register value is then copied to the specified

register.#

RCL (Recall).  When executed (CTRL-I) and followed by a storage

register number (00 to 99), a copy of the specified register value is

placed in the X register.#

Storage Register Arithmetic.  Arithmetic can be performed on the

contents of a storage register with the result placed in the storage

register. To do this, execute STO (SHIFT-I) followed by an arithmetic

operator (+, -, *, /), followed by a storage register number (00 to

99). For example, pressing SHIFT-I / 21 divides the value in register

21 by the value in X and stores the result in register 21.#

LAST OP

The LAST OP (Last Operation) register displays the most recently

performed operation.#

Clearing the Registers

CLRG (Clear Registers, CTRL-P) clears the contents of all storage

registers, the stack, and LAST X to zero.#