DESCRIPTION:

MEMBRAIN.EXE creates a file named 'MEMBRAIN.SYS', a DOS device driver

for an emulated disk drive configured to your specifications. DOS will

assign to the drive the next available drive letter. For example, if your

system has two diskette drives (A & B), then the emulated drive will be

drive C. The emulated drive will function identically to a hardware drive

with a few exeptions. First, this drive need not and connot be formatted.

Second, when you turn the system off, the drive ceases to exist. Be sure

to copy any files that must be saved to a hardware drive before power off.

Third, it's fast!!!

SYSTEM REQUIREMENTS:

An IBM PC or XT with at least 128K of memory and DOS 2.x operating

system.

INSTALLATION:

1) Enter on the DOS command line:

MEMBRAIN  disk size,sector size,cluster size,directory entries,path

disk size - The requested size of the emulated drive in Kbytes.

An error will be returned if the resulting drive leaves

less than 64K of usable memory. If this parameter is omitted

the default disk size is 160K.

sector size - The size of each logical sector in bytes. Sector

size should be a multiple of 128 and will be rounded up to

the next 128 byte multiple. For example, if the parameter is

entered as 150, then the actual sector size will be 256 bytes.

The default is 512 bytes per sector.

cluster size - The number of sectors per allocation unit. An error

will be returned if the number of bytes per cluster exceeds

2048. The general rule for adjusting sector size and cluster

size is to use small sector sizes and 1 sector per cluster for

small drives. Larger sector and cluster sizes will provide more

usable disk space per Kbyte of disk size and faster operation

on larger drives. The default is 1 sector per cluster.

directory entries - The maximum number of filenames that the disk

directory can hold. Smaller drives should be configured with

fewer directory entries. The more directory space is allocated,

the less disk space will be left for storing file data. This

parameter will be adjusted upward as necessary to a multiple

of sector size. The default is 64 directory entries.

path - The drive and directory path in which MEMBRAIN.SYS is to be

created. This parameter is entered in the form 'd:\path'. If

drive is omitted, then the default drive will be used. If path

is omitted, then the current directory will be used. In any

case, MEMBRAIN.SYS must be available when DOS is booted.

NOTES:

1) If MEMBRAIN.EXE is called from a batch file, then the DOS

'IF ERRORLEVEL' batch file command can be used to test for

successful completion. A non-zero return code will be passed

if an error occurs.

2) To create a CONFIG.SYS file:

Enter on the DOS command line:

COPY CON: CONFIG.SYS

DEVICE=MEMBRAIN.SYS

Press F6 and then <ENTER>

NOTES:

1) CONFIG.SYS must be on the diskette used to boot DOS.

2) If you already use a CONFIG.SYS file, then edit it to

include the statement 'DEVICE=MEMBRAIN.SYS'.

3) Re-boot DOS either by powering the system off and then back on, or

with the key sequence <Ctrl>-<Alt>-<Del>.

Press ENTER to continue: #owering the