TI-PC and the IBM-PC ROM BIOS Conversion List

*****************************

**         I   CRT         **

*****************************

TI PC                               IBM PC

CRT DSR..........INT 49             Video I/O..........INT 10

Section 4.10 of the Tech Ref Man     Page 43 of the IBM BIOS Listing

If the AH Register is:              If the AH Register is:

AH=00  Null Function               AH=00  Set Mode

AH=01  Set Cursor Type             AH=01  Set Cursor Type

Both computers use the CX Register to set the Cursor Type. The

IBM can not disable the Cursor Blink.

AH=02  Set Cursor Position         AH=02  Set Cursor Position

DH=COLUMN Location of Cur.          DH=ROW Location of Cursor

DL=ROW Location of Cursor           DL=COLUMN Location of Cur.

NOTE: The ROW/COLUMN designation has changed and the TI PC doesn't need

to set the page number like the IBM PC does.

AH=03  Read Cursor Position        AH=03  Read Cursor Position

DH=COLUMN Location of Cur.          DH=ROW Location of Cursor

DL=ROW Location of Cursor           DL=COLUMN Location of Cur.

NOTE:  The ROW/COLUMN designation has changed and the CX Register is

initialized with the same data. The TI-PC does not set the page number.

AH=04  Null Function               AH=04  Read Light Pen Position

AH=05  Null Function               AH=05  Select Active Display Page

AH=06  Scroll Text UP              AH=06  Scroll Page UP

AL=0: Move Block                    AL=0:  Blank Entire Screen

AL>0: Copy Block                    AL>0:  No of lines blanked at the

bottom of the screen

BH,BL: Destination begin            BH:    Attribute used on a blank

(COL/ROW Location)                  line

BL:    Not used

CH,CL: COL/ROW Length of            CH,CL: ROW/COL of Upper Left

Block                               corner of Block

DH,DL: Source begin COL/            DH,DL: ROW/COL of Lower Right

ROW Location                        corner of Block

NOTE: The ROW/COLUMN designation is changed in the CX and DX Registers

AH=07  Scroll Text DOWN            AH=07  Scroll Page DOWN

Same as the Scroll UP

AH=08  Read Char & Attribute at    AH=08  Read Char & Attribute at

current Cursor position            current Cursor position

AH=09  Write Char & Attribute at   AH=09  Write Char & Attribute at

current Cursor position            current Cursor position

The AL, BL, and CX Registers are initialized the same on

both computers. The BH Reg is not used by the TI-PC.

AH=0A  Write Character only at     AH=0A  Write Character only at

current Cursor position            current Cursor position

The AL and CX Registers are the same for both computers

AH=0B  Null Function               AH=0B  Set Color Palette

AH=0C  Null Function               AH=0C  Write Dot

AH=0D  Null Function               AH=0C  Read Dot

AH=0E  Write ASCII Teletype        AH=0E  Write Teletype

AL: Character to write              AL: Character to write

BL: Foreground color (Graphics

BX Register is not used                 Mode)

BH: Display page in alpha mode

AH=0F  Null Function               AH=0F  Return current Video State

*****************************

**      II  KEYBOARD       **

*****************************

TI-PC                               IBM-PC

Keyboard DSR..........INT 4A        Keyboard I/O..........INT 16

Sect 4.12 of the Tech Ref Manual   Page 23 of the IBM BIOS Listing

If the AH Register is:              If the AH Register is:

AH=00  Read Keyboard INPUT         AH=00  Read Keyboard INPUT

Both Computers place the Character in the AL Register

AH=01  Read Keyboard STATUS        AH=01  Read Keyboard STATUS

Both Computers operate the same

AH=02  Read Keyboard MODE          AH=02  Return the current Shift Status

Mode is returned in the AL Register

AH=03  Flush Keyboard Buffer       AH=03  Null Function

AH=04  Keyboard OUTPUT             AH=04  Null Function

AH=05  Put Character into          AH=05  Null Function

Keyboard Buffer

*****************************

**      III  PRINTER       **

*****************************

TI-PC                               IBM-PC

Print DSR..........INT 4B           Print I/O..........INT 17

Section 4.13 of the Tech Ref Man   Page 42 of the IBM BIOS Listing

AH=00  OUTPUT Character in AL      AH=00  OUTPUT Character in AL

Both operate the same

AH=01  Initialize the Printer      AH=01  Initialize the Printer

Both operate the same

AH=02  Return Printer Status       AH=02  Read Printer Status

Both operate the same but the status bits are different

*****************************

**        IV  DISK         **

*****************************

TI-PC                               IBM-PC

Disk DSR..........INT 4D            Diskette I/O..........INT 13

Section 4.11 of the Tech Ref Man   Page 32 of the IBM BIOS Listing

If the AH Register is:              If the AH Register is:

AH=00  Reset Disk System           AH=00  Reset Disk System

Both reset the disk to a known state

AH=01  Return Status Code          AH=01  Read Status

Both operate the same and return the status in the AL Register

The following is a list of the Registers used in the

READ/WRITE/ VERIFY/FORMAT DSR:

DL - Drive Number (0-3)

DH - Track (Surface) Number (0-1)

CH - Cylinder Number (0-39)

CL - Sector Number (1-8)

AL - Number of Sectors

ES:BX - Segment:Offset of Buffer

AH=02  Read Sectors                AH=02  Read Sectors

AH=03  Write Sectors               AH=03  Write Sectors

AH=04  Verify Sectors              AH=04  Verify Sectors

AH=05  Format Track                AH=05  Format Track

For each of the READ/WRITE/VERIFY/FORMAT DSR's

the operation is the same for both computers

AH=06  Verify Data                 AH=06  Null Function

AH=07  Return ReTry Status         AH=07  Null Function

AH=08  Set Standard Disk           AH=08  Null Function

Interface Table (DIT)

for the unit

AH=09  Set DIT address for the     AH=09  Null Function

unit

AH=0A  Return DIT address for      AH=0A  Null Function

the unit

AH=0B  Turn OFF Diskette Drive     AH=0B  Null Function

Motors

*****************************

**  V  TIME-OF-DAY CLOCK   **

*****************************

TI-PC                               IBM-PC

Time-of-Day Clock DSR..INT 4E       Time of Day Call.....INT 1A

Section 4.9 of the Tech Ref Man    Page 77 of the IBM BIOS Listing

If the AH Register is:              If the AH Register is:

AH=00  Set the Date                AH=00  Read the current clock settings

BX: The Date                        AL: 24 Hour Indicator

CX: High portion of count

DX: Low portion of count

AH+01  Set the Time                AH+01  Set the Current Clock

Both operate the same

AH=02  Get the Date & Time         AH=02  Null Function

*****************************

**   VI  8259 INTERRUPTS   **

*****************************

TI-PC                               IBM-PC

8259 INT (TIMER)........INT 43     8259 INT (0) Timer......INT 8

8259 INT 6 (Disk Cont)..INT 46     8259 INT (6) Diskette...INT E

8259 INT 7 (Keyboard)...INT 47     8259 INT (1) Keyboard...INT 9

8259 INT Vectors 0, 1, 2, 4, 5     8259 INT Vectors 2, 3, 4, 5, 7

are not used on the TI PC           are not used on the IBM PC

*****************************

** VII  INTERRUPT VECTORS  **

*****************************

The Interrupt Vectors 0-4 are the same on both Computers

*****************************

**       VIII  OTHER       **

*****************************

Keyboard Print Screen.....INT 5E   Print Screen..........INT  5

Memory Size...............INT 66   Memory Check..........INT 12

System Config Call........INT 4F   Equipment Check.......INT 11

The Following TI Interrupts are not used on the IBM PC

Speaker DSR..................................INT 40

Clock & Analog Interface Board...............INT 4C

System Configuration.........................INT 4F

Fatal Software Error.........................INT 50

Restart Timing Event.........................INT 51

Cancel Timing Event..........................INT 52

Service Interfacing Subroutine...............INT 53

Activate Task Subroutine.....................INT 54

Reserved for Future Use......................INT 55 & 56

CRT Mapping Vector...........................INT 57

System Timing, 25ms..........................INT 58

Common Interrupt Exit Vector.................INT 59

System Timing, 100ms.........................INT 5A

Keyboard Mapping Vector......................INT 5B

Keyboard Program Pause Key Vector............INT 5C

Keyboard Program Break Key Vector............INT 5D

Keyboard Queuing Vector......................INT 5F

ROM Pointers.................................INT 60 - 65

The Following IBM Interrupts are not used by the TI PC

RS232 I/O Call...............................INT 14

Cassette I/O Call............................INT 15

ROM Basic Entry Code.........................INT 18

Boot Strap Loader............................INT 19

Get Control on Keyboard Break................INT 1B

Get Control on Timer Interrupt...............INT 1C

Pointer to Video Initialization Table........INT 1D

Pointer to Diskette Parameter Table..........INT 1E

Pointer to Table for Graphics Char Gen.......INT 1F

**************************************************

**     IX  DOS CALLS & HIGH LEVEL LANGUAGES     **

**************************************************

The DOS Interrupts (INT 20-3F) and Function Calls are identical on

both computers. For High Level languages (FORTRAN, PASCAL, etc) the

object code should be compatable as long as you did not interface with

assembly language. If you did, convert your assembly language code.